##### Department of Electrical and Computer Engineering North South University

- Md Sadikur Rahman Sadeed    2112251042

- Md.Esak Ali        1921789042

- Ahanaf Tahomid  1831920642

- Md. Iftekhar Mahmood  2013525042 	 

CSE440 
sec : 01

# Robot Path Planning and Control Simulation Using A Algorithm and Unicycle Model*

## Table of Contents

- Introduction

- System Architecture
2.1 Robot Class
2.2 Node Class

- Unicycle Motion Model

- A Path Planning Algorithm*

- Proportional Control for Trajectory Following

- Obstacle Avoidance and Replanning

- Visualization Techniques

- Experimental Results

- Strengths and Limitations

- Future Work and Improvements

- Conclusion

- References

## 1. Introduction

Autonomous mobile robots require efficient and reliable navigation strategies to move within known or unknown environments. This report analyzes a Python-based simulation that integrates an A* path planning algorithm with a unicycle robot control model to enable real-time obstacle-aware navigation in a grid environment. This simulation serves as a foundational implementation for robot localization and trajectory following.

## 2. System Architecture

### 2.1 Robot Class

The Robot class encapsulates core components:

- Unicycle motion model implementation

- Pose (x, y, θ) control

- A* algorithm for planning

- Real-time visualization

- Obstacle setting and sensing

python

CopyEdit

def set_pose(self, x, y, theta):

self.x = x

self.y = y

self.theta = theta

### 2.2 Node Class

The Node class supports A*:

- Stores the current grid position

- Maintains references to parent nodes

- Calculates and compares path cost values (f = g + h)

## 3. Unicycle Motion Model

The robot follows the unicycle kinematic model:

x˙=vcos⁡(θ),y˙=vsin⁡(θ),θ˙=ω\dot{x} = v \cos(\theta), \quad \dot{y} = v \sin(\theta), \quad \dot{\theta} = \omegax˙=vcos(θ),y˙​=vsin(θ),θ˙=ω

Where:

- vvv: linear velocity

- ω\omegaω: angular velocity

- θ\thetaθ: orientation

These equations are discretized using Euler’s method for real-time simulation:

python

CopyEdit

self.x += v * math.cos(self.theta) * self.dt

self.y += v * math.sin(self.theta) * self.dt

self.theta += omega * self.dt

## 4. A Path Planning Algorithm*

A* is a heuristic-based search algorithm. It ensures the shortest path from start to goal while avoiding obstacles.

### Heuristic Function:

Uses Euclidean distance:

h(n)=(xgoal−xn)2+(ygoal−yn)2h(n) = \sqrt{(x_{\text{goal}} - x_n)^2 + (y_{\text{goal}} - y_n)^2}h(n)=(xgoal​−xn​)2+(ygoal​−yn​)2​

### Grid Movement:

Allows 8 directions (diagonal + orthogonal).

### Implementation Flow:

- Initialize start and goal nodes.

- Use priority queue (heapq) for open list.

- Expand neighbors with cost updates.

- Stop on reaching goal node.

- Reconstruct path using parent references.

## 5. Proportional Control for Trajectory Following

The robot computes control inputs at each time step to track the planned path using a proportional controller:

### Control Strategy:

- Angular velocity ω\omegaω is proportional to angle error.

- Linear velocity vvv is allowed only when the heading is close to the desired direction.

python

CopyEdit

if abs(angle_diff) < 0.5:

v = min(Kp_v * distance, 0.5)

else:

v = 0

### Tuning:

- Kpv=2.0Kp_v = 2.0Kpv​=2.0

- Kpω=1.0Kp_{\omega} = 1.0Kpω​=1.0

## 6. Obstacle Avoidance and Replanning

The environment includes static obstacles placed in the grid. The robot continuously checks for blockages and triggers replanning when necessary.

### Path Blockage Detection:

python

CopyEdit

def path_blocked(self, path):

for point in path:

if self.obstacle_map[int(point[0]), int(point[1])]:

return True

This ensures the robot adapts if obstacles are added or detected during runtime.

## 7. Visualization Techniques

Matplotlib is used for real-time 2D visualization:

- Green arrow: robot with direction

- Blue path: planned trajectory

- Black squares: obstacles

- Red circle: goal

python

CopyEdit

plt.arrow(self.x, self.y, dx, dy, head_width=0.3, head_length=0.2, fc='g', ec='g')

The visualization updates every control cycle, allowing real-time analysis.

## 8. Experimental Results

### Simulation Setup:

- Grid size: 15x15

- Start: (0, 0, 0)

- Goal: (10, 10)

- Obstacles: [(5, 5), (3, 6), (3, 8), (4, 2), (8, 8), (8, 4)]

### Observations:

- Path was successfully computed

- Robot followed path smoothly

- Real-time replanning triggered when path blocked

## 9. Strengths and Limitations

### Strengths:

- Combines motion model with global planning

- Efficient A* algorithm

- Modular structure

- Visualization assists in debugging

### Limitations:

- No dynamic obstacle detection

- A* produces jagged paths

- Proportional controller can be unstable under noise

- No SLAM (Simultaneous Localization and Mapping)

## 10. Future Work and Improvements

To make the system more robust and scalable:

- Path Smoothing: Use Bezier curves or spline interpolation

- Dynamic Obstacle Handling: Add sensors and reactive control

- Better Controllers: Implement PID or MPC

- Localization Module: Incorporate sensor fusion

- 3D Visualization: Upgrade for better realism

## 11. Conclusion

This simulation integrates A* path planning with a unicycle model and simple control logic, offering a foundational prototype for mobile robotics. It demonstrates effective path computation and tracking while navigating around static obstacles. The modular nature of the code allows future extensions in control, sensing, and planning domains.

## 12. References

- LaValle, S. M. (2006). Planning Algorithms. Cambridge University Press.

- Thrun, S., Burgard, W., & Fox, D. (2005). Probabilistic Robotics. MIT Press.

- Python Matplotlib Documentation

- Robotics Toolbox Theory — Peter Corke